Instruction of Black-Comet spectro radiometer

Kazunari Nozue
April 19, 2011
tel: 752-8086, emal:knozue@ucdavis.edu

beta version 0.1

Purpose: To measure fluence rate (micro E).

Equipments:

Black-Coment (StellarNet, Black CXR-sR-50 (813-855-8687)) , a spectroradiometer

SpectraWiz (StellarNet)

Starting Equipments:

1. Connect Black-Comet to Toshiba laptop computer through USB cable (use right USB slot which can provide power to Black-Comet and green LED on Black-Comet turns on). If you use left USB slot, green LED on Black-Comet does not turn on.

2. Connect a detector-cable to Black-Comet. Keep the soft black cover (on the end of detector-cable) in the plastic bag.

3. Turn on Toshiba Computer and choose “Black-Comet” account.

4. 

An outline of procedure.

1. Adjust integration time and scan numbers for averages at SCOPE mode

2. Take dark spectra at RAD mode

3. Measure sample spectra at RAD mode.

4. Store sample spectrum data.

5. Analyze the spectrum data (eg. R/FR ratio, PAR, total fluence, draw spectrum graph)

Detailed protocols

1. Adjust integration time and scan numbers for averages at SCOPE mode

You must be sure that you are setting the correct settings in SCOPE mode prior to each measurement.  Whether the aperture is installed on the cosine receptor or not, you will need to set the integration time in SCOPE mode first.  The best procedure is as follows:

1) First, after opening the software -make sure the calibration coefficients are entered properly (they should save after the first entry)

2) Start in SCOPE mode (Fig. 1) and point the receptor at your sample.  This is when [image: image1.png]| S— —
pechraWTz Spectrometer Oporating Sofevare (6] 2010w Stolar =
AL e @) 2] ] W] ]
RsEGREEY  viatis _ Wi~ n*2jm
Channel Lumen per square foot (footcandies)
Multi-Graph e ¢ )
st S forRaont ndLous Focares
. j’a ° Setup Range for Watt and Rfiux measurement.
9152 Z’me Setup Compensation for CR2 Aperture
.
:
:
v
Recydebin |

w” .«

Y untited - Paint | 8 smal chanber_o... | ) smal chanber Lo

IPEE T QLR RIABAAD s




you’ll want to adjust your integration time, smoothing, averages, etc.

Fig.1. A magenta arrow indicates Scope mode and a green yellow indicates slide bar for adjusting integration time. Integration time (ms) is shown on the bottom of spectrum window (indicated by an yellow arrow).

3) Increase or decrease the integration so that you get the peak to the top of the screen, without being cut-off (saturating). 

 (Fig. 2)

4)      THEN, select View > Radiometer > MicroMoles per square meter per second (Fig3). 

[image: image6.png]— L 3

pectraWiz Spectrometer Operating re (c) vovaw.Stellarl

[7ic scip Tven sors 0o b 50 Abs T Rl

] A | g m| 2 o] ] 1 [[E R

fe—
e

, siap ot

2 Wave ratio
vscde b
Zaom

Grapn race 25 »

ss1sd

=

Tran Rad

NI

341ms Avgt Sl S50 Teoff Xe3 Cul

300 00 500 500 700 00

Wavalsnzth in Nanomatsrs

SCOPE> Wave 722 $7am Fixi050 VAE20961500 Tima341ms Aoz S 550 Teoff 503 Gl Y
Y soeent-pant  spectawiz

BEILIDORVALALD L W 1asem




 (Fig. 3)

This will change mode from SCOPE to RAD. If you have an warning message (Fig. 4), click “cancel” to keep current integration time.

[image: image2.png]traWiz Spectrometer Operating Software (c) 2010 www.StellarNet.us

HAl 8o ¢la o 2 e urll YPF:76.67 PPE:0.83
¥ PAR: 86.063

Confirm

9 ) Configuring SpectroRadiometer @ 238ms.

RangeError indicates too much ight input:
1. F possible back away from source or
s 2. Tnstal calbrated zperture or

3. Decresse ntegraton tme.

Take dark spectrum to zero ightlevell
Cancel to keep current ntegraton tme,

=

350 00 50 500

!Jl Wavelenzth in Nanometers
T TN D> Wave72287om Fori030 Vi 0037 Tomeirms Avid Socl S50 Teof 2 03 Cirl 7

IPEE T Q0L RRIABAAD 6w

For Help, cick Help Topics on the Help Menu.
5 start

Y soemn2i-pant | ) smal chamber .. | ) smal chamber_




 (Fig. 4)

If you OK, please start over from step2.

Unplug optical cable and place a black cap on light meter. Take a dark spectrum by clicking the black light bulb button (you will see the baseline go down to zero).(Fig. 5 and 6)

[image: image3.png]1 SpectraWiz Spectrometer Operating Software (c) 2010 www.StellarNet.us =JE3 e .
Fie Setup View Apps Disk Help Sco Abs Tran Rad L.
Al gle el g s 2r kel = YPF:9.85 PPE:0.75 »
¢ PAR: 10.212 M B
Oste oded Ostepcure Taen | Dimenso
bee w9t v weax7e -
N W x7e
N W x7e £
[pge  1/14/2011 1:48PM 2048 x 7€ \é
e 1192011 v W x7e 0 \?
N W x7e
. e W x7e
e 1192011 1M W x7e
e W x7e
e W x7e
e 1192011 nseem W x7e
N W x7e
e W x7e
e W x7e
e 1192011 n7RM W x7e
s e W x7e
e 19 zzem W x7e
hee  9ni 2y W x7e
- e 11901 2220w B 7E
- e i
fce 113367 i
fce 113377 i
e vsinizazm i
e vasionzem i
5
.
: % % % E" 4
Wavelearth in Nanomaters
RAD> Wave72287am Fix 1050 Val 0.020 Timer769ms Aved Sl S50 Teoff Xe3 Chil \




 (Fig. 5, before taking a dark spectrum)

[image: image4.png]1 SpectraWiz Spectrometer Operating Software (c) 2010 www.StellarNet.us =JE3 e .
Fie| Setp Vew Apps Dsk Fep  Sco Abs Tran Red L.
Al gle el g s 2r kel || YPF:001 PPE:0.60 »
¢ PAR: 0.0181 M B
Oste oded Ostepcure Taen | Dimenso
bee w9t v weax7e -
N W x7e
N W x7e £
[pge  1/14/2011 1:48PM 2048 x 7€ \é
e 1192011 v W x7e 0 \?
N W x7e
. e W x7e
e 1192011 1M W x7e
e W x7e
e W x7e
e 1192011 nseem W x7e
N W x7e
e W x7e
e W x7e
e 1192011 n7RM W x7e
s e W x7e
e 19 zzem W x7e
hee  9ni 2y W x7e
- e 11901 2220w B 7E
- e i
fce 113367 i
fce 113377 i
e vsinizazm i
e vasinzem Desx7E
s Y Y weax7e
5
: p P kS £ 4
Wavelearth in Nanomaters
RAD> Wave72287am Fix 1050 Val 0.000 Time769ms Aved Sl S50 Teoff Xe3 Chil \

)| O Yoaeenaspane TS EE SO, DR BB




 (Fig. 6, after taking a dark spectrum)

5) Unplug a black cap on light meter and connect an optical fiber to begin measuring.

Typical spectrum of fluorescence lamp is shown in Fig. 7 (please take notes on exposure time just in case).

[image: image5.png]j . S

i

WA SpectraWiz Spectrometer Operating Software (c) 2010 veviw.StellarNet.us o

Rl scup Viw [Apps) Dk o || S0 Abs] Tren R
HAl Bl o gl gl @ 2w kfl | YPF210.00 PPE:0.77
¢ PAR: 231.80 m

A

a

300 00

500 500 700 500 500
Wavalsnzth in Nanomatsrs

[FAD> Wave72287m 251050 Val: 0.525 TimerdSms Ave Sl 350 Teoff SNAD Xt Chil

BH s %

| Jﬂ Options v

=] € 9 4cf2a-Clipboard v x

B 1| [@earmt

Cick an tem to paste - A

&) i Documents and
Setinge ek cametpesc .
0.5 Wiaves 722 67

&) Fie: Documents and /
Setinga lac CometDeskt
0.5 Wiaves 722 67

) e Documents and
Setinge ek cametpesc
0.5 Wiaves 722 67

To display this task pane agai, cick
Cipboard on the it menu or

4

Y sreens -pant

MicosoftExce

BN IRCBRVAAAD L 22w





6) If you want to change ranges of PAR (default is 400 nm to 700 nm), you can go to 

View > Radiometer > Setup Range for Watt and R flux measurement

You will be asked input lowest and highest wavelength (nm).

7) You can save the spectrum data in a text file by File > Save > Sample.

8) Excel or any text editors can read the saved .IRR file. (.IRR stands for irradiance). 

Enjoy! 

