Microarray Wash

Pipet 2mL of 10% Triton X-102 into both wash buffers.

Mix carefully by inverting 5-6 times.

Remove inner and outer caps and replace with spigot provided

*Remove any traces of contaminating material from staining dishes by running copious amount of Milli-Q water through the slide racks and stir bars

*Set up wash apparatus in the hood

Wash 1: 1 min

Wash 2: 1 min

Do wash for up to 8 slides at once

All stir bars should be set to medium speed!

When removing slide rack from Wash 2, minimize droplets on slides.

Scan slides immediately, or else store in orange slide boxes in nitrogen purge box (in the dark).

Scanning the Slides

Place slide in slide holder with active side facing up and barcode opposite end of rear ledge.

Place ozone-barrier slide cover on top of the array with the pocket of the cover facing the array.

Place assembled slide holders into scanner carousel.

Verify the following scanner settings:

Scan region Scan Area (61x21.6 mm)

Scan resolution (um) 5

5um scanning mode Single Pass

eXtended Dynamic range (selected)

Dye channel Red&Green

Green PMT XDR Hi 100% Lo 10%

Red PMT XDR Hi 100% Lo 10%

Prefix 1 is set to Instrument Serial Number

Prefix 2 is set to Array Barcode

Verify that the Scanner status in the main window says Scanner ready

Click Scan Slot m-n on the Scan Control

*m represents the Start slot and n represents the End Slot

Red PMT

Prefix 1 is set to Instrument Serial Number

Prefix 2 is set to Array Barcode

Verify that the Scanner status in the main window says Scanner ready

Click Scan Slot m-n on the Scan Control

*m represents the Start slot and n represents the End Slot